Applied Biosystems PowerUp SYBR Green qPCR

for algal symbiont community profiling

Written by M. Studivan

Updated: 12 Jun 2026 M. Studivan

- Prior to first use, rehydrate lyophilized primers using NFW or TE to make 100 µM working stocks. Store at -20ºC after rehydrating.

- Optional: Create working aliquots of primers with volumes depending on the total number of plates needed, in order to minimize freeze-thaw cycles of the 100 µM working stocks.

- 1) Thaw DNA extracts, primer aliquots, and PowerUp SYBR Green master mix on ice. Turn on QuantStudio 3.

- 2) Prepare plate layout and master mix recipe according to the template “qPCR algal symbiont community profiling plate template.xlsx.”

- 3) Quick spin all reagents once thawed. Prepare master mixes according to calculated recipe, adding water first, then SYBR Green master mix, and finally primers. The number of master mixes depends on the number of primer combinations you have (e.g., for host, B, C, and D, you will have four master mixes). Mix by gentle inversion, then quick spin. Keep master mixes on ice. The recipe will make enough master mix for two plates.

- 4) Pipet 9 µL of prepared master mixes into the wells of a white PCR plate in a color-changing, frozen plate rack, with two rows per primer combination. Pipet the bead on the back wall of the PCR plate in each well.

- 5) Pipet 1 µL of DNA extracts into each well, with one column per sample. Pipet the bead on the front wall of the PCR plate in each well, so you don’t have to switch pipet tips. Plate 1 will fit 12 samples, and Plate 2 will fit 11 samples; add 1 µL of NFW into column 12.

- 6) Once finished loading, gently tap the PCR plate on the benchtop to bring the beads to the bottom of the wells. Use qPCR optical film to seal the plate with the rubber roller. Quick spin the prepared PCR plate in the plate centrifuge (blue lab).

- 7) Insert the prepared PCR plate into the QuantStudio 3, with well A1 oriented to the top left of the heat block (the beveled corner of the plate will be in the bottom right).

- 8) Open the qPCR program template in the QuantStudio software (HostBCD_Samples1-12 for plate 1, HostBCD_Samples1-11_NTC). Select Run to start the program

- 9) While running, check the plot to verify fluorescence signals starting after the second cycle. Once completed, save the experiment file and export the results as an .xlsx.

qPCR Consumables Prep:

- 1.5 mL tubes for primer aliquots (1 per primer)

- 1.5 mL tubes for master mixes (1 per primer combination)

- White qPCR plates (2 per master mix recipe)

- Optical qPCR film (2 per master mix recipe)